Escali Alimento
Calibration Procedures

- Before turning on the scale, hold down the Tare, Backlight, and Kg/Lb buttons at the same time

- While holding down the three buttons, press the On/Off button

- The numbers 55555 will show on the screen; release the three buttons you are holding down, now press the Back Light button

- The screen will now display random numbers; you will now press the Tare button

- The message 2Ero will display for one second, and then displays 6000g

- Gently place 6000g on the platform, and press the Tare button

- The message CAL will display on the screen for a second, and then will display PASS if calibration is successful. (EEEEE will display if calibration was incorrectly done.)

Calibration complete